(Specifier Note:  The purpose of this guide specification is to assist the Specifier in correctly specifying security transaction windows and their installation.

The Specifier needs to edit this guide specification to fit the needs of each specific project.  References have been made within the text of the specification to MasterFormat section numbers and titles. The Specifier needs to coordinate these numbers and titles with sections included for the specific project.

Throughout the guide specification, there are Specifier Notes to assist in the editing of the file.  Brackets have been used to indicate when a selection is required. Contact an Insulgard representative for further assistance with appropriate product selections)

- SECTION 08 56 53 – SECURITY WINDOWS

- Insulgard SLV Manual Vertical Sliding Transaction Windows

GENERAL

SECTION INCLUDES

Vertical Sliding Transaction Windows

ACTION SUBMITTALS

Refer to Section [01 33 00 Submittal Procedures] [Insert section number and title].

Product Data: For each type of window

Shop Drawings:  Include plans, elevations, sections, details, anchorage to other work.

Samples:  For each exposed finish.

INFORMATION SUBMITTALS

Product Test Reports:  Indicating compliance with requirements

Warranty:  Sample of finish warranty

CLOSEOUT SUBMITTALS

Refer to Section [01 78 00 Closeout Submittals] [Insert section number and title].

Cleaning Instructions

DELIVERY, STORAGE AND HANDLING

Refer to Section [01 60 00 Product Requirements] [Insert section number and title].

Protect windows in accordance with AAMA CW-10 "Care and Handling of Architectural Aluminum from Shop to Site" until Substantial Completion.

WARRANTY

(Specifier Note: The 5 year finish warranty applies to the Class I anodic finishes and the 10 year applies to the 70% PVDF coating finish.)

Finish Warranty: Manufacturer’s warranty against deterioration of factory finishes for the period of [5] [10] years from the date of Substantial Completion.

PRODUCTS

(Specifier Note: Product information is proprietary to Insulgard Security Products.  If additional products are required for competitive procurement, contact Insulgard, Inc. for assistance in listing competitive products that may be available.)

MANUFACTURED UNITS

Basis of Design:  SLV Manual Vertical Sliding Transaction Window by Insulgard Security Products; Phone 800.624.6315; website www.insulgard.com

Subject to compliance with requirements, manufacturers of products of equivalent design may be acceptable if approved in accordance with [Section 01 25 00 Substitution Procedures] [Insert section number and title].

Description

(Specifier Note: Head and jamb sizes are dependent on the size of the unit. First options for small units less than 40 inches wide and 72 inches tall, second options apply for units up to 120 inches wide by 96 inches tall.

1.	Factory fabricated window constructed from either 6105-T5 or 6005-T5 extruded  aluminum with fixed and sliding transparent bullet resistant panels.

Jambs:  [(Small unit) 2-1/4 inch by 4 inch extruded aluminum] [(Large unit) 3 inch by 9 inch extruded aluminum with internal steel counter balance.]

Head:  [(Small unit) 6 inch by 4 1/2  inch  extruded aluminum with internal overhead counterbalance]   [(Large unit) 1 inch by 8 inch extruded aluminum]

Base: [plastic laminate clad particleboard] [stainless steel] [solid surface]:

Deal Tray:  Recessed stainless steel  [with bullet-trap]

Wimensions:  [Insert Custom Sizes]

Hardware: Manufacturer standard guide blocks, handles, counter balances, stops, mounting brackets, locks, and cables.

Locking: [manual plunger lock] [power operated plunger lock with power supply – 12 volt, 120 volt power source by others required.]

FABRICATION

Cope corners the full depth of framing; assemble and dress smooth.

Tolerances: All joints and connections shall be tight, providing hairline joints and true alignment of adjacent members

Factory fabricate windows

ALUMINUM FINISH

Factory-applied finish:

(Specifier Note:  SELECT the project specific finish from the following. Baked Enamel may also be available but may require minimum quantities.)

[Clear Anodic Finish]: Architectural Class I, clear coating AA-M10C22A41 Mechanical Finish Chemical Finish: etched, medium matte; 0.70 mils minimum complying with AAMA 611 "Voluntary Specification for Anodized Architectural Aluminum"

[Color Anodic Finish]: Architectural Class I, color coating AA-M10C22A42/A44 Mechanical Finish Chemical Finish: etched, medium matte; 0.70 mils minimum complying with AAMA 611 "Voluntary Specification for Anodized Architectural Aluminum".

Color: Dark Bronze.

[PVDF-Based Coating]: Fluoropolymer finish containing minimum 70 percent PVDF resins, in accordance with AAMA 2605 “Voluntary Specification, Performance Requirements and Test Procedures for Superior Performance Organic Coatings on Aluminum Extrusions and Panels”

Coats:  [two] [three] [coat system,

Color:  [Insert color] [custom] [to be selected from manufacturer's full color range].

GLAZING

Glazing Material: Ballistic Resistant Level [1] [2] [3] in accordance with UL 752 – Testing for Ballistic Resistance.

- (Specifier Note: SELECT glazing based on threat level and architect’s preference.

[Level 1]:1-1/4 inch acrylic

[Level 2]: 1-3/8 inch acrylic

[Level 3]: [1-1/4 inch acrylic] [1-1/4 inch SP1250 Lexgard]

ACCESSORIES

Fasteners:  Manufacturer recommended security fasteners.  Utilize fastener that require two types of tools to operate fasteners.

Sealants:  As indicated in Section 07 92 00 Joint Sealants.

Bituminous Paint:  Cold-applied, asphalt-mastic paint complying with SSPC-Paint 12 requirements except containing no asbestos; formulated for 30-mil thickness per coat.

EXECUTION

PREPARATION

Verify field dimensions of opening prior to fabrication of transaction window.

Coordinate structural requirements to ensure proper attachment and support.

INSTALLATION

Install transaction windows in accordance with manufacturer's recommendations and approved shop drawings.

Where dissimilar metals will be in contact, protect against galvanic action by painting contact surfaces with primer or by apply sealant or tape recommended in writing by window manufacturer.  When aluminum will contact concrete or masonry, protect against corrosion by painting contact surfaces with bituminous paint.

Provide required support and securely fasten and set windows plumb, square, and level without twist or bow.

CLEANING

Clean security window surfaces after installation, avoiding damage to finishes.  Remove excess glazing and sealant compound and dirt.

PROTECTION

Protect transaction windows damage during construction operations. If damage occurs, remove and replace as required to provide windows in their original, undamaged condition.

END OF SECTION